Methods of Trouble Shooting Common Failures

| Common FAIL. | Possible cause | The solution |
| --- | --- | --- |
| 1. Power Indicate not on | 1. No power 2. Ind. fail | 1. See if power Is on. 2.Change power indicator |
| 2. Trip of Circuit leak switch | 1. Machine circuit leak. 2. Parts & elet. Parts. broken | 1. Check Circuit leak of the machine, 2. See of C leak switch (ELB-S) is ok |
| 3. Wrong Indicate on Inverter | Refer to Inverter Manual |  |
| 4. Abnormal indication on Temperature device | Refer to Temperature control Manual |  |
| 5. Main machine can not be started(PC control) | - No power - Switch not on - Ele. Loop failure - Fuse blow | - see if power is on - see if C leak SW is on - See if cont loop SW is on - See if PC input term. Is open - See if fuse blows and base breaks - See if PC “POWER LED”is on |
| 6.Longitudinal seal not strong | - Adj of Long. Seal temp is not good - Contact of Long. Seal pressing roller is not good or pressure is too low - Long. Seal is not hot totally - Packing film quality is too poor | - adjust T as per packing film & packing speed - see if bearing of contact roller breaks and adjust its pressing force - check heat loop of long. Seal & replace thermal couple and heater - substitution of good packing film |
| 7.End seal not good | - Temp adj is not good - End seal adj is unsuitable - Quality of packing film is too poor - Heater is not hot | - Adj T as per quality of packing film - Adj ecc by ref to end sealer adj - Substitution of good film - check heat loop of end seal & replace thermal couple and heater clean brush |
| 8.Long. seal unstable after packing film going through bag forming device | - Packing film too narrow - Bag forming device size unsuitable - Posit of adj roller & bag forming device entrance is unsuitable - Lower tubing conveyor and bag forming device bottom are not on a plane - spindle brake is not well adjusted - H & B of article being packed are uneven and press of tubing conveyor is uneven - packing film roller is wound unevenly in tightness at both ends | - packing film shall be 30-50mm wider than articles in girth - adj or replace bag former - readjust as per empty bag operation - Re-adj of height of lower tubing conveyor - adjust brake belt - Improve quality of art. Being packed, widen packing film, use conveyor belt with upper & lower sponge and often check and mend - Replace packing film |
| 9.Bag length indicate unstable | - adj of sensor not good - tooth chain variable speed case fail - adj handle loose | - set space between to sensors & gear tip at 0.5-1mm - refer to tooth chain variable speed case op. manual & see if tension of tooth chain case is properly relaxed - after adj of bag Length value |
| 10.Tracking abnotmal | - bag length set too long or too short - printing quality of colored marks of film too poor - sensitivity of PE cell too H. or too L. - Tension on various parts of packing film too H. or too L. film del uneven - electric failure | - set bag length properly - replace packing film - adj. sensitivity repeatedly until resp is right. Or change it - elim packing film shake - tracking AC contactor failure (open) or tacking motor failure troubleshoot or replace them |
| 11.End seal gear teeth skip | Gear teeth skip is caused by film being wound round upper end sealer or by article lifting upper end sealer | Remove art. On end sealer, loosen end seal pressing handle, open upper cover at end seal gear, then lift upper end seal shaft and rplace gear. Replace power lock if it is not in space |

Refer to the following list for PC error alarm

List of error alarm

Ind. light on    Ind. Light flashing   Ind. Light off

| State after error | Error items | CPU UNIT LED STATE Power Run alarm error | Wrong inf of programming dev ind | Treatment |
| --- | --- | --- | --- | --- |
| Error causing operation stop | Power fail |  |  | Check supply volt and supply wiring |
|  | CPU err (over 130ms on monitoring timer) |  |  | Switch on under “programming” cond & it is necessary to recheck program |
|  | Memory error |  | Memory ERR | Check program,mod The unfit & reenter the Program Check for correctness of the dial switch. SET Check EPROM installation Check battery insertion Perform ERR REM OP After mod. |
|  | End inst omitted |  | NO END INST | Write in END INST |
|  | I/O bus error |  | I/O bus error | See if conn cable BEL units falls off, see if SET SW of I/O unit & conn cable are in the same dire. Conduct ERRREM OP after modification |
|  | JMP overflow |  | JMP OVER | Mod. Log of program JMP-JME to below 8 |
| Error not causing op stop | Bat erro. Scan overtime (monitoring timer 100-300ms) |  | BARRLOW | Check for bat. Insert. Use new bat. |
|  |  |  | SCAN TIME OVER | Recheck program |